SECTION 01 35 29
SAFETY AND HEALTH ON WLSSD PROJECTS
PART 1 – GENERAL

1.01 DESCRIPTION OF WORK


A. Initiate, maintain, and supervise safety and health precautions and programs in connection with the work as specified in the Contract Documents.  Take necessary precautions for the safety of, and provide the necessary protection to prevent damage, injury or loss to persons on the work site or those who may be affected by the work.  Failure to comply with the safety and health requirements of this Section and Section 00 70 00 – STANDARD GENERAL CONDITIONS OF THE CONSTRUCTION CONTRACT shall be cause for work site eviction and/or termination of contract at OWNER’s discretion. 

1.02 RELATED WORK ELSEWHERE
A. Submittals are delineated in Section 01 33 00 – SUBMITTALS


1.03 SUBMITTALS


A. Health and Safety Program – CONTRACTOR shall submit, prior to mobilization, one hard copy and one electronic copy of the company’s Health and Safety Program.  The Health and Safety Program shall include, but not be limited to, the safety policies needed to prevent damage, injury, or loss to all persons on the work site or who may be affected by the work.   The Health and Safety Program shall be applicable to all CONTRACTOR and SUBCONTRACTOR personnel.   

B. Electrical Safety Plan - The Health and Safety Program shall include an Electrical Safety Plan which includes written certification by an authorized representative of the CONTRACTOR and each SUBCONTRACTOR that all personnel have had training applicable to the individual’s assigned work.  All personnel doing electrical work have had to be trained in the most current version of NFPA 70E.  The certification included in the Electrical Safety Plan shall at a minimum include the date(s) of the training, location of the training, training provider/instructor and a list of personnel who attended the training.     

C. OSHA DART and TRC Ratings – Incorporate written certification of the CONTRACTOR’S OSHA DART and TRC ratings for the past three years into the Health and Safety Program. 

D. WLSSD Hot Work Permit – Submit the Hot Work Permit upon completion of the work. 

E. WLSSD Hot Work Permit for Spark Producing Equipment – Submit the Hot Work Permit for Spark Producing Equipment upon completion of the work. 

F. WLSSD Confined Space Entry Permit (Form CS-2) – Submit Form CS-2 upon cancellation of the permit
1.04 APPLICABLE PUBLICATIONS


A. Western Lake Superior Sanitary District Confined Space Entry Policy 

B. Western Lake Superior Sanitary District Hazardous Energy Control Procedure

1.05  STANDARDS AND CERTIFICATIONS

A. The following standards shall be followed while executing the work required by the Contract Documents:  

1. Safety and Health Regulations for Construction, promulgated by the Secretary of Labor under Section 107 of the Contract Work Hours and Safety Standards Act, as set forth in Title 29, C.F.R.  Copies of these regulations may be obtained from Labor Building, 14th and Constitution Avenue N.W., Washington, DC 20013. 

2. Federal Occupational Safety and Health Act, as amended.
PART 2 – PRODUCTS 


(NOT USED)
PART 3 – EXECUTION
3.01 DOCUMENTATION AND NOTIFICAITON

A. Safety Program – Document the Health and Safety Program in accordance with the requirements of  Paragraph 1.03 of this Section and Section 01 33 00 – SUBMITTALS.   

B. Notification of Start of Work – Contact Process Control to notify the operator that the CONTRACTOR will be on site completing the work required in the Contract Documents before starting any work on site.  Process Control can be contacted at the following phone number: (218) 740-4801 (or extension #301 from in plant phones). 

C. No Smoking Policy – Notify employees that smoking is not permitted on District property. 

D. Personnel List – Maintain a list of personnel working on the site.  The list should include documentation that each person has been trained according to the CONTRACTOR’S Safety Program and has read and understands the requirements of this Section. 

E. Notification of Confined Space Entry – Notify the designated District Project Representative prior to a confined space entry. 

F. Notification of Accident – Notify the designated District Project Representative of any and all accidents or incidents on the work site or pertaining to the work which have caused an injury. 

G. Hot Work Permit – Obtain a Hot Work Permit for electric or gas welding, cutting, brazing, or similar flame or spark producing operations that will be conducted anywhere in District facilities or on District property. 

H. Hot Work Permit for Spark Producing Equipment – Obtain a Hot Work Permit for Spark Producing Equipment for use of electric or other spark producing equipment while in use in a Hazardous (Classified) location. 

I. Confined Space Entry Permit – Complete a Confined Space Entry Permit in accordance with the WLSSD Confined Space Entry Policy for all entries into permit required confined spaces anywhere in District facilities or on District property    

3.02 GENERAL 

A. Designate a site safety representative that will be available at all times while the work is in progress. 

B. When a conflict exists between the CONTRACTOR’S Safety Program policies and the Contract Documents, follow the most stringent requirement. 

C. Assume full responsibility for injuries to persons and for damages to property and services of the District, and for claims, losses or expenses which may in any way arise out of or result from the CONTRACTOR’S performance of this contract unless caused by the negligence or willful misconduct of the District’s agents, or employees.  Indemnify and save the District harmless from such claims, losses or suits for injuries, death or damage and from liens, losses, expenses or claims of any sort arising therefrom and shall defend on behalf of the District any suit brought against the District for any such damage, injury or death.  Reimburse the District for attorneys’ fees and for other expenses incurred by the District in connection with or as a result of any such suit. 

D. Do not use District equipment unless otherwise specified. 

E. CONTRACTOR personnel shall not tamper with or alter District electrical equipment except as shown on the drawings or approved by the OWNER.  CONTRACTOR shall provide proper electrical plugs or adapters to it District receptacles.  Proper welding equipment plugs for District receptacles are Appleton, 4 pole, 4 wire, Style 600 VAC, 60 amp, Cat no. ACP 6044 BC or equivalent. 

3.03 SAFETY AND HYGIENIC PRECAUTIONS 

A. Hygienic Precautions – Require personnel working on the District’s collection facilities to observe proper hygienic precautions including washing properly, and not eating or smoking except after washing hands as personnel could be exposed to wastewater and refuse containing disease-producingorganisms. 

B. Safety Precautions– Take measures to make sure that personnel observe proper safety precautions when working on the District’s facilities.  The District’s wastewater collection, treatment and solid waste facilities can be extremely hazardous in terms of potential accident-causing situations.  Some safety precautions include: 

1. Fire extinguishers have been placed throughout the plant and at pump stations for anyone’s use.  Fire doors within the District facilities shall remain closed at all times. 

2. All aisles, stairways, passageways, exits and access areas to buildings and equipment shall be kept open and passable. 

3. Oil, water and other liquid spills shall be cleaned up immediately or covered with absorbent material. 

4. All compressed gas cylinders shall be stored and used in an upright, vertical position in properly designated racks.  They shall be secured by a chain to prevent them from falling over.   

C. Potential Hazards  - Some potential hazards present in the District wastewater collection, treatment and solid waste facilities include: 

1. Solvents, gasoline, and other hazardous materials enter the treatment plant, pump stations and conveyance system with incoming sewage, and therefore, certain areas are hazardous to open flame, sparks, or unventilated occupancy. 

2. Some areas of the wastewater treatment plant (oxygen production and dissolution areas of building 2 and 3) may have enriched oxygen, which greatly increases the risk of fire.  Use of flame or spark producing equipment is not allowed unless the air is constantly monitored for elevated oxygen levels. 

3. Sodium Hypochlorite (NaOCl) is stored in building 23 at the wastewater treatment plant and used for disinfection.  This is a corrosive material (pH>12) and will cause severe burns if it comes in contact with skin or clothing.  If contaminated with acid, it will release toxic chlorine gas.  Sodium hypochlorite will react with certain metals (such as iron, zinc, and tin) to give decomposition products that include oxygen, which can develop great pressures when confined between valves. 

4. Sodium Bisulfite (NaHSO3), which may release sulfur dioxide gas, is stored in building 6 at the wastewater treatment plant and used for dechlorination.  This chemical can be extremely hazardous and extreme caution should be used when working around this chemical. 

5. Ferric Chloride (FeCl3) is stored in building 19 at the wastewater treatment plant and is added to anaerobically digested sludge to control hydrogen sulfide and phosphorous levels.  This chemical is very corrosive (pH<1)
6. Digester gas (approximately 65% Methane (CH4), 35% Carbon Dioxide (CO2)) is produced in building 19 and present in piping located in buildings 10 and 11at the waste water treatment plant.  Digester gas is flammable, and has the potential of being explosive. 

7. Noise – CONTRACTOR shall note that all areas of District facilities have been surveyed to determine noise levels.  Length of exposure has been posted outside of these work areas.  CONTRACTOR’s shall wear hearing protection in these areas, as indicated on postings. 

D. Protective Clothing and Equipment – Require personnel working on District property to wear safety glasses, steel toed shoes, and hard hats at all times.  Require personnel working on District property in areas where they could be exposed to wastewater or residuals to wear proper protective equipment including: gloves, masks, eye protection, and protective body clothing. 

3.04 CONFINED SPACE ENTRY 

A. Confined space entry requirements in accordance with OSHA 1910.146, MN Rule 5207.0300 through 5207.0310 and the WLSSD Confined Space Entry Policy shall be followed anytime CONTRACTOR’S personnel are in a confined space as defined by OSHA 1910.146. 

B. All tanks, channels, vessels, manholes, pipelines, wet wells and other permitted confined spaces which contain or have contained wastewater or biosolids may not contain enough oxygen to support life.  Atmosphere in these areas shall be monitored and ventilated prior to and during entry. 

C. CONTRACTOR shall follow the WLSSD Confined Space Entry Policy and use the WLSSD 

D. Confined Space Entry Permit (Form CS-2) for all entries into permitted confined spaces.    

3.05 HOT WORK PERMIT 

A. Obtain a hot work permit before conducting hot work activities. There are special procedures that are to be followed for hot work that is to be conducted in a confined space.  These procedures are outlined in the District’s confined space entry policy which should be reviewed before attempting entry (also refer to lockout/tag-out procedures), and include: 

1. All necessary work and emergency equipment shall be gathered prior to entry. 

2. After calibration of monitor, the area shall be tested prior to entry by trained personnel. 

3. Before any work begins, set up and start adequate ventilation equipment.  

4. A water hose of minimum 5/8” dia. shall be at the site, and shall be used to wet down all areas within the confined space. 

5. Close all mud valves and other ports of potential hazards. 

6. DO NOT do any digging of sand, gravel, or other debris as it may produce dangerous atmospheres. 

7. Before performing hot work, retest the air throughout the confined space, and at the pointwhere the work is to be performed. 

8. Conduct continuous monitoring of the confined space.   

9. Maintain wet surfaces throughout the confined space. 

10. Open flames are to be used only for welding, cutting, brazing, etc.  DO NOT use open flames for cleaning debris off surfaces, or other non-hot work procedures.  

B. The District’s Hot Work Permit form is attached to this Section. Complete a permit for each hot work activity that is conducted as part of the Contract Documents.  Keep the permit on file at the location of the hot work, until the hot work is completed.  Submit the permit to the Field Representative upon completion of the hot work or expiration of the permit.


3.06 HOT WORK PERMIT FOR SPARK PRODUCING EQUIPMENT

A. Obtain a hot work permit for spark producing equipment before using electrical equipment in a Hazardous (Classified) location for which it is not approved. 

END OF SECTION
[ATTACH HOT WORK PERMIT]
	Safety and Health on WLSSD Projects
	Western Lake Superior Sanitary District
WLSSD Project Number XXXXXX

	01 35 29 - 2
	WLSSD Bid Number XXXX


	Western Lake Superior Sanitary District
Project Name XXXXXX
	Safety and Health on WLSSD Projects

	WLSSD Bid Number XXXX
	01 35 29 - 1



